Table 1. Calibration and Experimental Contract

| Component | Frozen evidence or setting | Scope and interpretation |
| --- | --- | --- |
| Demand timing | Sao Paulo Olist purchase timestamps; 15,540 orders | Proxy for demand timing, not last-mile release timestamps. |
| Spatial demand | 12 fixed calibration-only abstract zones | Zone weights are fitted on calibration data; platform origin remains an engineering assumption. |
| Weather | Sao Paulo GFS 06 UTC forecasts paired with ERA5 actual 10 m wind | 20-minute matched replay; forecast-minus-actual speed bias = 0.488 m/s. |
| Weather gates | P50 = 0.630 m/s; P80 = 1.163 m/s; primary P90 = 1.479 m/s | P90 fixed before the 60-scenario primary evaluation; P80 is a boundary condition. |
| Energy | External DJI Matrice 100 public mission-energy envelope | Transfer envelope only; no claim of same-city or target-aircraft telemetry. |
| Policy factors | Planning access: always vs event-triggered; inertia: fixed vs adaptive | Five-policy evaluation also includes nonfactorial myopic reference. |
| Safety | Shared ForcedSafetyTrigger validates the released first slot | Unsafe task forces safe replan or deterministic hold/swap/defer for every policy. |
| Primary evaluation | 60 shared Sao Paulo order-weather holdout replications; common random numbers | Scenario-level paired Student-t CI is primary; paired bootstrap and order-level GLMM are diagnostics. |

Note. Input distributions are empirically informed. VIP labels, platform movement, W_3, and H_s remain scenario or governance parameters rather than observational estimates.